Co3

# EXPERIMENT NO. 5

## TITLE

Computer-Aided Design and Optimization of a Microwave Impedance-Matching Network and Frequency-Selective Filter

## 1. AIM

To design and simulate a microwave impedance-matching network and a frequency-selective filter using MATLAB, and to analyse reflection coefficient, return loss, input impedance, insertion loss, cutoff frequency, and percentage deviation.

## 2. SOFTWARE REQUIRED

MATLAB

## 3. MATLAB PROGRAM

clc;

clear;

close all;

%% EXPERIMENT 5

% Computer-Aided Design and Optimization of a Microwave

% Impedance-Matching Network and Frequency-Selective Filter

%% INPUT PARAMETERS

f0 = 10e9;                  % Design frequency: 10 GHz

Z0 = 50;                    % Reference impedance: 50 ohms

ZL = 75 + 1j*25;            % Load impedance: 75 + j25 ohms

fc = 10e9;                  % Filter cutoff frequency: 10 GHz

f = linspace(8e9,12e9,401); % Frequency sweep: 8-12 GHz

fGHz = f/1e9;

c = 3e8;

%% QUESTION A: IMPEDANCE MATCHING NETWORK

% Reflection coefficient before matching

GammaL = (ZL-Z0)/(ZL+Z0);

GammaBefore = abs(GammaL);

RLBefore = -20*log10(GammaBefore);

% Cancel the load reactance with a series reactance

Xcancel = -imag(ZL);

Zcancelled = ZL + 1j*Xcancel;

% Quarter-wave transformer impedance

Zt = sqrt(Z0*real(Zcancelled));

% Quarter-wave transformer length at 10 GHz

lambda0 = c/f0;

Lqw = lambda0/4;

% Frequency-dependent matching network response

% Ideal transmission-line model

Xseries = Xcancel*(f/f0);

Zadjusted = ZL + 1j*Xseries;

beta = 2*pi*f/c;

ZinMatch = zeros(size(f));

for k = 1:length(f)

theta = beta(k)*Lqw;

ZloadAdjusted = Zadjusted(k);

ZinMatch(k) = Zt * ...

(ZloadAdjusted + 1j*Zt*tan(theta)) / ...

(Zt + 1j*ZloadAdjusted*tan(theta));

end

% Reflection coefficient after matching

GammaAfterComplex = (ZinMatch-Z0)./(ZinMatch+Z0);

GammaAfter = abs(GammaAfterComplex);

% Return loss after matching

RLAfter = -20*log10(max(GammaAfter,1e-12));

% Before-matching response

GammaBeforeF = GammaBefore*ones(size(f));

RLBeforeF = RLBefore*ones(size(f));

% Index corresponding to 10 GHz

[~,idx0] = min(abs(f-f0));

%% DISPLAY QUESTION A RESULTS

fprintf('\n========================================\n');

fprintf('QUESTION A: IMPEDANCE MATCHING NETWORK\n');

fprintf('========================================\n');

fprintf('Load impedance = %.2f + j%.2f ohms\n',...

real(ZL),imag(ZL));

fprintf('Reference impedance = %.2f ohms\n',Z0);

fprintf('Reflection coefficient before matching = %.4f\n',...

GammaBefore);

fprintf('Return loss before matching = %.2f dB\n',...

RLBefore);

fprintf('Series reactance for cancellation = %.2f ohms\n',...

Xcancel);

fprintf('Quarter-wave transformer impedance = %.2f ohms\n',...

Zt);

fprintf('Quarter-wave transformer length = %.2f mm\n',...

Lqw*1000);

fprintf('Input impedance after matching at 10 GHz = %.2f + j%.2f ohms\n',...

real(ZinMatch(idx0)),imag(ZinMatch(idx0)));

fprintf('Reflection coefficient after matching = %.6f\n',...

GammaAfter(idx0));

fprintf('Return loss after matching = %.2f dB\n',...

RLAfter(idx0));

%% QUESTION B: MICROWAVE LOW-PASS FILTER

% Third-order Butterworth low-pass filter

N = 3;

ratio = f/fc;

% Ideal transmission magnitude

S21 = 1./sqrt(1+ratio.^(2*N));

% Ideal lossless reflection magnitude approximation

S11 = sqrt(max(1-S21.^2,0));

% Convert magnitudes into dB

S21dB = 20*log10(max(abs(S21),1e-12));

S11dB = 20*log10(max(abs(S11),1e-12));

% Find -3 dB cutoff frequency

targetdB = -10*log10(2);

idxCut = find(S21dB <= targetdB,1,'first');

if isempty(idxCut)

fcSim = NaN;

elseif idxCut == 1

fcSim = f(1);

else

f1 = f(idxCut-1);

f2 = f(idxCut);

y1 = S21dB(idxCut-1);

y2 = S21dB(idxCut);

fcSim = f1 + ...

(targetdB-y1)*(f2-f1)/(y2-y1);

end

% Percentage deviation

cutoffDeviation = abs(fcSim-fc)/fc*100;

% Find point closest to 10 GHz

[~,idxFc] = min(abs(f-fc));

% Insertion loss and return loss at cutoff

insertionLoss = -S21dB(idxFc);

returnLossFilter = -S11dB(idxFc);

%% DISPLAY QUESTION B RESULTS

fprintf('\n========================================\n');

fprintf('QUESTION B: MICROWAVE LOW-PASS FILTER\n');

fprintf('========================================\n');

fprintf('Filter type = Third-order Butterworth low-pass\n');

fprintf('Reference impedance = %.2f ohms\n',Z0);

fprintf('Required cutoff frequency = %.2f GHz\n',fc/1e9);

fprintf('Calculated -3 dB cutoff frequency = %.4f GHz\n',...

fcSim/1e9);

fprintf('Cutoff frequency deviation = %.4f %%\n',...

cutoffDeviation);

fprintf('Insertion loss at cutoff = %.2f dB\n',...

insertionLoss);

fprintf('Return loss at cutoff = %.2f dB\n',...

returnLossFilter);

%% OUTPUT PARAMETER TABLE VALUES

fprintf('\n========================================\n');

fprintf('OUTPUT PARAMETERS FOR LAB RECORD\n');

fprintf('========================================\n');

fprintf('Load reflection coefficient = %.4f\n',GammaBefore);

fprintf('Input impedance after matching = %.2f + j%.2f ohms\n',...

real(ZinMatch(idx0)),imag(ZinMatch(idx0)));

fprintf('Return loss before matching = %.2f dB\n',RLBefore);

fprintf('Return loss after matching = %.2f dB\n',RLAfter(idx0));

fprintf('Filter cutoff frequency = %.4f GHz\n',fcSim/1e9);

fprintf('Cutoff frequency deviation = %.4f %%\n',cutoffDeviation);

fprintf('Insertion loss at cutoff = %.2f dB\n',insertionLoss);

%% COMBINED OUTPUT GRAPHS

figure(1);

clf;

set(gcf,'Color','w','Visible','on');

% GRAPH 1: REFLECTION COEFFICIENT

subplot(2,2,1);

plot(fGHz,GammaBeforeF,'r--','LineWidth',2);

hold on;

plot(fGHz,GammaAfter,'b','LineWidth',2);

plot([10 10],[0 1],'k:');

grid on;

xlabel('Frequency (GHz)');

ylabel('Reflection Coefficient |Gamma|');

title('A: Reflection Coefficient');

legend('Before Matching','After Matching','10 GHz',...

'Location','best');

ylim([0 1]);

% GRAPH 2: RETURN LOSS

subplot(2,2,2);

plot(fGHz,RLBeforeF,'r--','LineWidth',2);

hold on;

plot(fGHz,RLAfter,'b','LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('A: Return Loss');

legend('Before Matching','After Matching',...

'Location','best');

% GRAPH 3: FILTER S-PARAMETERS

subplot(2,2,3);

plot(fGHz,S11dB,'r','LineWidth',2);

hold on;

plot(fGHz,S21dB,'b','LineWidth',2);

plot([8 12],[targetdB targetdB],'k:');

plot([10 10],[-60 5],'k--');

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

title('B: Filter S-Parameters');

legend('|S11|','|S21|','-3 dB Level','10 GHz',...

'Location','best');

ylim([-60 5]);

% GRAPH 4: FILTER CUTOFF FREQUENCY

subplot(2,2,4);

plot(fGHz,S21dB,'b','LineWidth',2);

hold on;

plot([8 12],[targetdB targetdB],'r--');

plot([10 10],[-60 5],'k:');

grid on;

xlabel('Frequency (GHz)');

ylabel('|S21| (dB)');

title('B: Filter Cutoff Frequency');

legend('Transmission','-3 dB Level','Required 10 GHz',...

'Location','best');

ylim([-60 5]);

% Force MATLAB to update and display the figure

drawnow;

fprintf('\nAll four graphs generated successfully.\n');

## 4. CALCULATIONS AND OUTPUT

# QUESTION A: MICROWAVE IMPEDANCE-MATCHING NETWORK

### Given Data

Load impedance, (Z_L=75+j25\ \Omega)

Characteristic impedance, (Z_0=50\ \Omega)

Operating frequency, (f_0=10\ \text{GHz})

Frequency sweep = 8–12 GHz

## Calculation 1: Load Reflection Coefficient

Formula:

Reflection Coefficient = (Load Impedance − Characteristic Impedance) / (Load Impedance + Characteristic Impedance)

Given:

Load Impedance, ZL = 75 + j25 Ω

Characteristic Impedance, Z0 = 50 Ω

Substitution:

ΓL = [(75 + j25) − 50] / [(75 + j25) + 50]

ΓL = (25 + j25) / (125 + j25)

Magnitude of reflection coefficient:

|ΓL| = √(25² + 25²) / √(125² + 25²)

|ΓL| = √1250 / √16250

|ΓL| = 35.355 / 127.475

Answer: |ΓL| = 0.2774

## Calculation 2: Return Loss Before Matching

Formula:

Return Loss (RL) = −20 log₁₀ |ΓL|

Substitution:

RL = −20 log₁₀(0.2774)

RL = −20 × (−0.5570)

Answer: RL = 11.14 dB

Therefore, the theoretical return loss before matching is 11.14 dB.

## Calculation 3: Input Impedance After Ideal Matching

For ideal impedance matching:

Input Impedance, Zin = Characteristic Impedance, Z0

Substitution:

Zin = 50 + j0 Ω

Answer: Zin = 50 Ω

Therefore, the ideal matched input impedance is 50 Ω.

## Calculation 4: Reflection Coefficient After Ideal Matching

Formula:

Γin = (Zin − Z0) / (Zin + Z0)

Substitution:

Γin = (50 − 50) / (50 + 50)

Γin = 0 / 100

Answer: |Γin| = 0

Therefore, the ideal matched network produces zero reflection.

## Calculation 5: Return Loss After Ideal Matching

Formula:

Return Loss (RL) = −20 log₁₀ |Γin|

Substitution:

RL = −20 log₁₀(0)

Since log₁₀(0) is undefined, the return loss approaches infinity for an ideal, perfectly matched network.

Answer: RL → ∞ dB

In a practical simulation, the return loss is generally finite because perfect matching is not always achieved.

### Output Table – Question A

S. No.; Parameter; Theoretical Value. 1; Load impedance; (75+j25\ \Omega). 2; Characteristic impedance; (50\ \Omega). 3; Operating frequency; 10 GHz. 4; Reflection coefficient before matching; 0.2774. 5; Return loss before matching; 11.14 dB. 6; Ideal matched input impedance; (50+j0\ \Omega). 7; Reflection coefficient after ideal matching; 0. 8; Return loss after ideal matching; Approaches infinity..

# QUESTION B: FREQUENCY-SELECTIVE FILTER

### Given Data

Characteristic Impedance, Z0 = 50 Ω

Theoretical Cutoff Frequency, fc(theory) = 10 GHz

Start Frequency, f(start) = 8 GHz

Stop Frequency, f(stop) = 12 GHz

Filter Type = Second-order Butterworth low-pass filter

## Calculation 1: Frequency Sweep Range

Formula:

Frequency Range = Stop Frequency − Start Frequency

Substitution:

Frequency Range = 12 − 8

Answer: Frequency Range = 4 GHz

## Calculation 2: Centre Frequency

Formula:

Centre Frequency = (Start Frequency + Stop Frequency) / 2

Substitution:

Centre Frequency = (8 + 12) / 2

Centre Frequency = 20 / 2

Answer: Centre Frequency = 10 GHz

## Calculation 3: Insertion Loss at 10 GHz

For a second-order Butterworth low-pass filter:

Insertion Loss (IL) = 10 log₁₀ [1 + (f / fc)⁴]

Given:

Operating Frequency, f = 10 GHz

Cutoff Frequency, fc = 10 GHz

Substitution:

IL = 10 log₁₀ [1 + (10 / 10)⁴]

IL = 10 log₁₀ (1 + 1)

IL = 10 log₁₀ (2)

Answer: IL = 3.01 dB

## Calculation 4: Cutoff Frequency

For a Butterworth filter, the cutoff frequency occurs when the insertion loss is approximately 3 dB.

At f = 10 GHz:

IL = 10 log₁₀ [1 + (10 / 10)⁴]

IL = 10 log₁₀ (2)

IL = 3.01 dB

Answer: Simulated Cutoff Frequency, fc(sim) ≈ 10 GHz

## Calculation 5: Percentage Deviation of Cutoff Frequency

Formula:

Percentage Deviation = |fc(sim) − fc(theory)| / fc(theory) × 100

Substitution:

Percentage Deviation = |10 − 10| / 10 × 100

Percentage Deviation = 0 / 10 × 100

Answer: Percentage Deviation = 0%

## Calculation 6: Bandwidth

The designed filter is a low-pass filter. Therefore, its bandwidth is determined by its cutoff frequency rather than by subtracting two cutoff frequencies.

Formula:

Bandwidth = Cutoff Frequency − 0

Substitution:

Bandwidth = 10 − 0

Answer: Bandwidth = 10 GHz

Within the specified simulation sweep of 8–12 GHz, the frequency span is 4 GHz. This is the sweep range, not the full low-pass filter bandwidth.

Note: This formula applies to a band-pass filter. For a low-pass or high-pass filter, report the applicable cutoff frequency instead.

### Output Table – Question B

S. No.; Parameter; Value. 1; Characteristic impedance; 50 Ω. 2; Theoretical cutoff frequency; 10 GHz. 3; Start frequency; 8 GHz. 4; Stop frequency; 12 GHz. 5; Frequency sweep range; 4 GHz. 6; Centre frequency; 10 GHz. 7; Simulated (S_{11}); From MATLAB output. 8; Simulated (S_{21}); From MATLAB output. 9; Simulated cutoff frequency; From MATLAB graph. 10; Insertion loss; Calculated from (S_{21}). 11; Bandwidth; From cutoff frequencies, if applicable. 12; Percentage deviation; Calculated using Calculation 5.

## COMBINED MATLAB OUTPUT

Fig. 1: Combined MATLAB simulation output of the microwave impedance-matching network and frequency-selective filter.

## 5. RESULT

The microwave impedance-matching network and frequency-selective filter were analysed using MATLAB. The theoretical reflection coefficient before matching was calculated as 0.2774, and the corresponding return loss was 11.14 dB. For ideal impedance matching, the input impedance was obtained as 50 Ω with zero reflection. The frequency-selective filter was evaluated over the 8–12 GHz range, and its simulated S-parameters, cutoff frequency, insertion loss, and percentage deviation were identified for comparison with theoretical values.