Co1

# EXP. 2 — QUESTION A

## TRANSMISSION-LINE/TWO-PORT S-PARAMETERS

### 1. AIM

To model a 50 Ω microwave two-port network of length 50 mm and simulate its S-parameters (S11, S12, S21 and S22) over the frequency range of 8–12 GHz with a center frequency of 10 GHz. To calculate return loss, insertion loss and transmission efficiency at 10 GHz and compare the simulated values with the ideal 50 Ω matched reference.

### 2. SOFTWARE USED

MATLAB

### 3. MATLAB CODE

clc;

clear;

close all;

%% EXPERIMENT 2: MICROWAVE TRANSMISSION-LINE S-PARAMETERS

% INPUT PARAMETERS

f0 = 10e9;              % Operating frequency: 10 GHz

Z0 = 50;                % Reference impedance: 50 ohms

ZL = 50;                % Load impedance: 50 ohms

fstart = 8e9;           % Start frequency: 8 GHz

fstop = 12e9;            % Stop frequency: 12 GHz

L = 50e-3;              % Network length: 50 mm

c = 3e8;                % Speed of light (m/s)

% MODEL ASSUMPTIONS

% The assignment does not specify line loss or characteristic impedance.

% Assume a slightly mismatched, lossy line to demonstrate frequency response.

Zline = 55;             % Assumed line characteristic impedance (ohms)

loss10_dB = 1.5;        % Assumed line insertion loss at 10 GHz (dB)

%% FREQUENCY SWEEP

f = linspace(fstart, fstop, 4001);

N = length(f);

S11 = zeros(1,N);

S21 = zeros(1,N);

S12 = zeros(1,N);

S22 = zeros(1,N);

%% CALCULATE S-PARAMETERS USING ABCD MATRIX

for k = 1:N

% Frequency-dependent attenuation

% Assumes attenuation varies approximately with sqrt(f)

alpha_dB = loss10_dB * sqrt(f(k)/f0);

alpha = alpha_dB * log(10)/20;  % Np for voltage-wave amplitude

beta = 2*pi*f(k)/c;             % Phase constant

gamma = alpha + 1j*beta;

% Transmission-line ABCD parameters

A = cosh(gamma*L);

B = Zline*sinh(gamma*L);

C = sinh(gamma*L)/Zline;

D = cosh(gamma*L);

% Convert ABCD to S-parameters, equal 50-ohm port references

den = A + B/Z0 + C*Z0 + D;

S11(k) = (A + B/Z0 - C*Z0 - D)/den;

S21(k) = 2/den;

S12(k) = 2*(A*D - B*C)/den;

S22(k) = (-A + B/Z0 - C*Z0 + D)/den;

end

%% FIND VALUES AT 10 GHz

[~,idx10] = min(abs(f-f0));

S11_10 = S11(idx10);

S21_10 = S21(idx10);

S12_10 = S12(idx10);

S22_10 = S22(idx10);

S11_dB_10 = 20*log10(max(abs(S11_10),eps));

S21_dB_10 = 20*log10(max(abs(S21_10),eps));

% Return loss and insertion loss

RL_10 = -S11_dB_10;

IL_10 = -S21_dB_10;

% Transmission efficiency

eta_10 = abs(S21_10)^2*100;

%% FIND BEST OPERATING FREQUENCY

S11_dB = 20*log10(max(abs(S11),eps));

S21_dB = 20*log10(max(abs(S21),eps));

% Best matching: minimum reflection magnitude

[~,idx_best_match] = min(abs(S11));

% Best transmission: maximum transmission magnitude

[~,idx_best_trans] = max(abs(S21));

f_best_match = f(idx_best_match);

f_best_trans = f(idx_best_trans);

%% DISPLAY INPUT PARAMETERS

fprintf('\n========== INPUT PARAMETERS ==========\n');

fprintf('Operating frequency : %.2f GHz\n',f0/1e9);

fprintf('Reference impedance : %.2f ohms\n',Z0);

fprintf('Frequency sweep     : %.2f to %.2f GHz\n',...

fstart/1e9,fstop/1e9);

fprintf('Network length      : %.2f mm\n',L*1e3);

fprintf('Load termination    : %.2f ohms\n',ZL);

fprintf('Assumed line Zc     : %.2f ohms\n',Zline);

fprintf('Assumed loss at 10 GHz: %.2f dB\n',loss10_dB);

%% DISPLAY OUTPUT PARAMETERS AT 10 GHz

fprintf('\n======= OUTPUT AT 10 GHz =======\n');

fprintf('S11 = %.5f %+.5fj\n',real(S11_10),imag(S11_10));

fprintf('S12 = %.5f %+.5fj\n',real(S12_10),imag(S12_10));

fprintf('S21 = %.5f %+.5fj\n',real(S21_10),imag(S21_10));

fprintf('S22 = %.5f %+.5fj\n',real(S22_10),imag(S22_10));

fprintf('\nS11 magnitude = %.2f dB\n',S11_dB_10);

fprintf('S21 magnitude = %.2f dB\n',S21_dB_10);

fprintf('Return loss   = %.2f dB\n',RL_10);

fprintf('Insertion loss= %.2f dB\n',IL_10);

fprintf('Efficiency    = %.2f %%\n',eta_10);

%% IDEAL MATCHED REFERENCE

fprintf('\n======= IDEAL 50-OHM REFERENCE =======\n');

fprintf('S11 = 0\n');

fprintf('S22 = 0\n');

fprintf('|S21| = 1 (0 dB)\n');

fprintf('|S12| = 1 (0 dB)\n');

fprintf('Return loss = Infinity\n');

fprintf('Insertion loss = 0 dB\n');

fprintf('Efficiency = 100 %%\n');

%% DISPLAY OPTIMUM FREQUENCIES

fprintf('\n======= OPTIMUM FREQUENCY =======\n');

fprintf('Minimum |S11| frequency = %.4f GHz\n',...

f_best_match/1e9);

fprintf('S11 at best match       = %.2f dB\n',...

S11_dB(idx_best_match));

fprintf('Return loss at match    = %.2f dB\n',...

-S11_dB(idx_best_match));

fprintf('S21 at best match       = %.2f dB\n',...

S21_dB(idx_best_match));

fprintf('\nMaximum |S21| frequency = %.4f GHz\n',...

f_best_trans/1e9);

fprintf('S11 at best transmission = %.2f dB\n',...

S11_dB(idx_best_trans));

fprintf('S21 at best transmission = %.2f dB\n',...

S21_dB(idx_best_trans));

fprintf('Return loss there        = %.2f dB\n',...

-S11_dB(idx_best_trans));

fprintf('Insertion loss there     = %.2f dB\n',...

-S21_dB(idx_best_trans));

%% PLOT ALL S-PARAMETERS IN ONE FIGURE

figure('Name','Experiment 2: S-Parameter Analysis',...

'NumberTitle','off');

subplot(2,2,1);

plot(f/1e9,S11_dB,'LineWidth',1.6);

hold on;

plot(f_best_match/1e9,S11_dB(idx_best_match),'ro',...

'MarkerFaceColor','r');

grid on;

xlabel('Frequency (GHz)');

ylabel('|S11| (dB)');

title('Input Reflection S11');

subplot(2,2,2);

plot(f/1e9,S21_dB,'LineWidth',1.6);

hold on;

plot(f_best_trans/1e9,S21_dB(idx_best_trans),'ro',...

'MarkerFaceColor','r');

grid on;

xlabel('Frequency (GHz)');

ylabel('|S21| (dB)');

title('Forward Transmission S21');

subplot(2,2,3);

plot(f/1e9,20*log10(max(abs(S12),eps)),'LineWidth',1.6);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S12| (dB)');

title('Reverse Transmission S12');

subplot(2,2,4);

plot(f/1e9,20*log10(max(abs(S22),eps)),'LineWidth',1.6);

grid on;

xlabel('Frequency (GHz)');

ylabel('|S22| (dB)');

title('Output Reflection S22');

sgtitle('50-mm Microwave Transmission Line: 8-12 GHz');

### 4. OUTPUT

Insert the MATLAB output screenshots showing the S-parameter curves over 8–12 GHz and the simulated values at 10 GHz.

Record the following parameters:

- S11: Input reflection coefficient

- S12: Reverse transmission coefficient

- S21: Forward transmission coefficient

- S22: Output reflection coefficient

- Return loss: RL = −20 log₁₀|S11|

- Insertion loss: IL = −20 log₁₀|S21|

- Transmission efficiency: η = |S21|² × 100%

### 5. RESULT

The 50 Ω microwave two-port network was simulated in MATLAB over the frequency range of 8–12 GHz. The S-parameters were obtained, and the return loss, insertion loss and transmission efficiency were evaluated at 10 GHz and compared with the ideal matched reference.