Co1

# EXP. 2 — QUESTION B

## OPTIMUM FREQUENCY ANALYSIS

### 1. AIM

To identify the optimum operating frequency from the 8–12 GHz S-parameter sweep by locating the minimum magnitude of S11 and maximum magnitude of S21. To record the corresponding S-parameters, return loss and insertion loss and compare them with the values at 10 GHz.

### 2. SOFTWARE USED

MATLAB

### 3. MATLAB CODE

clc;

clear;

close all;

%% QUESTION B: OPTIMUM OPERATING FREQUENCY

% Input parameters (same as Question A)

Z0 = 50;                % Reference impedance (ohms)

Zline = 55;             % Assumed characteristic impedance (ohms)

L = 50e-3;              % Line length (50 mm)

f0 = 10e9;              % Center frequency (10 GHz)

fstart = 8e9;           % Start frequency (8 GHz)

fstop = 12e9;           % Stop frequency (12 GHz)

c = 3e8;                % Speed of light (m/s)

loss10_dB = 1.5;        % Assumed loss at 10 GHz

%% Frequency sweep

f = linspace(fstart,fstop,4001);

N = length(f);

S11 = zeros(1,N);

S21 = zeros(1,N);

S12 = zeros(1,N);

S22 = zeros(1,N);

%% Calculate S-parameters

for k = 1:N

% Frequency-dependent attenuation

alpha_dB = loss10_dB*sqrt(f(k)/f0);

alpha = alpha_dB*log(10)/20;

% Propagation constant

beta = 2*pi*f(k)/c;

gamma = alpha + 1j*beta;

% ABCD parameters of lossy transmission line

A = cosh(gamma*L);

B = Zline*sinh(gamma*L);

C = sinh(gamma*L)/Zline;

D = cosh(gamma*L);

% ABCD to S-parameters

den = A + B/Z0 + C*Z0 + D;

S11(k) = (A+B/Z0-C*Z0-D)/den;

S21(k) = 2/den;

S12(k) = 2*(A*D-B*C)/den;

S22(k) = (-A+B/Z0-C*Z0+D)/den;

end

%% Magnitudes in dB

S11_dB = 20*log10(max(abs(S11),eps));

S21_dB = 20*log10(max(abs(S21),eps));

%% Find optimum frequencies

% Minimum input reflection

[~,idx_min] = min(abs(S11));

% Maximum forward transmission

[~,idx_max] = max(abs(S21));

f_min_S11 = f(idx_min);

f_max_S21 = f(idx_max);

%% Values at 10 GHz

[~,idx10] = min(abs(f-f0));

RL_10 = -S11_dB(idx10);

IL_10 = -S21_dB(idx10);

%% Values at optimum matching frequency

S11_best = S11(idx_min);

S21_at_match = S21(idx_min);

RL_best = -S11_dB(idx_min);

IL_at_match = -S21_dB(idx_min);

%% Values at optimum transmission frequency

S11_at_trans = S11(idx_max);

S21_best = S21(idx_max);

RL_at_trans = -S11_dB(idx_max);

IL_best = -S21_dB(idx_max);

%% Display results

fprintf('\n===== QUESTION B: OPTIMUM FREQUENCY =====\n');

fprintf('\nAt 10 GHz:\n');

fprintf('|S11| = %.4f dB\n',S11_dB(idx10));

fprintf('|S21| = %.4f dB\n',S21_dB(idx10));

fprintf('Return loss = %.4f dB\n',RL_10);

fprintf('Insertion loss = %.4f dB\n',IL_10);

fprintf('\nMinimum |S11|:\n');

fprintf('Optimum matching frequency = %.4f GHz\n',...

f_min_S11/1e9);

fprintf('S11 magnitude = %.4f dB\n',S11_dB(idx_min));

fprintf('S21 magnitude = %.4f dB\n',S21_dB(idx_min));

fprintf('Return loss = %.4f dB\n',RL_best);

fprintf('Insertion loss = %.4f dB\n',IL_at_match);

fprintf('\nMaximum |S21|:\n');

fprintf('Optimum transmission frequency = %.4f GHz\n',...

f_max_S21/1e9);

fprintf('S11 magnitude = %.4f dB\n',S11_dB(idx_max));

fprintf('S21 magnitude = %.4f dB\n',S21_dB(idx_max));

fprintf('Return loss = %.4f dB\n',RL_at_trans);

fprintf('Insertion loss = %.4f dB\n',IL_best);

%% Plot results and mark optimum frequencies

figure('Name','Question B: Optimum Frequency',...

'NumberTitle','off');

subplot(2,1,1);

plot(f/1e9,S11_dB,'LineWidth',1.8);

hold on;

plot(f_min_S11/1e9,S11_dB(idx_min),'ro',...

'MarkerFaceColor','r');

xline(10,'--k','10 GHz');

grid on;

xlabel('Frequency (GHz)');

ylabel('|S11| (dB)');

title('Minimum Reflection Frequency');

subplot(2,1,2);

plot(f/1e9,S21_dB,'LineWidth',1.8);

hold on;

plot(f_max_S21/1e9,S21_dB(idx_max),'ro',...

'MarkerFaceColor','r');

xline(10,'--k','10 GHz');

grid on;

xlabel('Frequency (GHz)');

ylabel('|S21| (dB)');

title('Maximum Transmission Frequency');

sgtitle('Optimum Frequency Analysis: 8-12 GHz');

### 4. OUTPUT

Insert the MATLAB output screenshots showing the S-parameter curves and the optimum frequency.

Record the following:

- Optimum frequency

- S11 at the optimum frequency

- S21 at the optimum frequency

- Return loss at the optimum frequency

- Insertion loss at the optimum frequency

- Comparison of these values with those at 10 GHz

### 5. RESULT

The optimum operating frequency was identified from the simulated S-parameter sweep by determining the minimum |S11| and maximum |S21|. The return loss and insertion loss at this frequency were compared with their values at 10 GHz to evaluate the matching and transmission performance of the network.